| Product name | 19-inch 4u rackmount chassis |
| --- | --- |
| Product Feature: |  |
| 450AS is a standard 19-inch 4U rackmount chassis with a height of 4U, made of high-quality galvanized steel with no horses. The structure design is novel, sturdy, compact and reasonable, and the installation and operation are convenient. One low-noise 1225 large fan can support two 5.25-inch optical drives, nine 3.5-inch hard drives or seven 2.5-inch hard drives, and supports ATX power supply. Products are widely used in industrial automation, communications, electric power, network security, intelligent transportation, video surveillance recording, medical, military and other industries. Industry/aerospace, self-service terminal, data storage, digital signage, AI intelligence, vehicle computer, 3C application, etc. |  |
| Technical Parameters: |  |
| Size | 482(W)*450(D)*176 mm(H) |
| Supported Motherboards | 12"* 10"(305 * 245MM) |
| Hard disk location | Support 3.5''9 or 2.5''7 (optional) |
| CD-ROM location | 2 5.25'' optical drives \ 1 floppy drive |
| Support power | Ordinary Desk-type ATX Power Supply， PS\2 Power Supply |
| Support fan | 1 front 1 12C iron mesh mute big fan |
| Expansion Slot | 7 full height PCI/PCIE straight slots |
| Panel configuration | USB2.0*2\power switch*1\restart switch*1 Blue keyboard switch*1 power indicator*1\hard disk indicator*1 Support slide rail: support |
| Product material | High-quality flowerless galvanized steel |
| Material Thickness | 1.2MM |
| Delivery time | 1 Week for sample ,2 Weeks for mass goods |
| Payment Terms | 30%TT Prepay 70%TT before shipment |